# Technical Details for Full Proposal

- Origin of the Proposal:

CBCT scans for tooth implantation assessment performs a vital role. These scans are able to present the three dimensional (3D) view of the captured images of tooth under consideration. The details present in these scans are able to check the quality of the bone and the tooth to be considered for  implantation process. These scans are able to generate the idea about the possibility of the happening of the successful implant. After scanning or generation, these CBCT images required the attention of an expert to mark the different locations of interest like Crest, IAN Canal, Nasal Floor etc. These locations further helps in the generation of idea abort the quantitative parameter i.e. Height, existing among these located points. These heights will be used next to extract the idea about the different levels of Lekholm & Zarb and Misch classifications. This basically summarize the overall report by introducing the qualitative parameters. The proposed system will be able to generate the automated medical reports for tooth implantation process rather than the manual procedure explained above. The input to the system will be considered as the CBCT (Cone Beam Computed Tomography) scans. The proposed system will reduce the requirement of sending these scans to the medical expert for the marking of different regions of interests. These regions further with the help of knowledge based system carried by the expert to conclude the implantation possibilities of the tooth under consideration. The major research questions addressed by the proposed system is the identification of upper and lower jaw from CBCT scans. Then the segmentation of tooth area and the bone area form these scans. With the help of artificial intelligence model these areas will be identified. For the quantitative assessment, different heights such as crest to sinus floor, crest to IAN floor etc. will be calculated. These quantitative values further will help in the qualitative analysis of tooth and bone by considering Lekholm & Zarb and Misch classifications. The major challenges involved in this work are the collection of CBCT scans, Segmentation of various region of interests, identification of the specific section of the CBCT data to make observations regarding the quantitative measures. AI based model is also required to deal with the automatic generation of reports. The automatic generation of medical report without the intervention of clinician or doctor from the CBCT dental scans is the novel aspect associated with the proposed system. The segmentation of specific point of interest such as crest, nasal floor, IAN canal etc. leads to the selection of appropriate level of also leads to the level of Lekholm & Zarb and Misch classifications. This further help in the generation of a summary report which will be easily understandable by the  patients and their family members.

# State of Art Review in the Proposed Subject

## 2.1	Review the status and highlight the significance of the proposed work in terms of:

## Hussaini et al. (2024) presented a quantitative analysis for the use of CBCT dental scans. Their study helped in case of assessing the bone quality and the requirement of ancillary procedure. They selected 200 random CBCT images from 4800 tooth implants. Authors found that in about 7% cases the CBCT scans provided a kind of impression that the bone quality was not good to initiate the implantation process. On the other hand with the limited information of OPG scans if the implantation process started by the experts then its consequences may lead to in-between aborting the ongoing treatment, requirement of alternate treatment, issues of patient behavior, lack of trust on doctors etc. [1]

## Saha & Nair (2023)	elaborated the role of CBCT imaging in case of dental implantation process. They presented the availability of panoramic view, axial view, cross sectional view, inferior alveolar canal and 3D reconstructed view in case of CBCT scans to analyze the different things in depth. This depth analysis is not possible earlier with OPG images. This in-depth information enable the clinicians to make a pro-theses  for implantation process. [2]

## Mikic et al. (2022) worked over elaborating the concept of bone density analysis with CBCT scans. Authors observed that the success rate of implant therapy is approximately 95%. The success rate depends upon the implant type, patient type and skill set of the clinician. They performed an experiment to get an idea about the bone quality by taking animal material such as pig ribs and bovine femur. In case of bovine femur, the average bone desity for lower jaw region was observed s 851.8 HU and the lowerest value observed as 442.9 HU. It ranges from 442.9 to1236.9 HU. The classification category observed as D1and D2 according to Misch classification. Pig rib was used for upper jaw with average value of 255.7. Authors observed that with the analysis of bone tissues, the prediction of the degree of primary stability of non-self tapping implant, if the classifications were under D1, D2 and Q1-Q3.[3]

## Xiong et al. (2019) Cone-beam computed tomography scan incidental discoveries that result in external referrals are examined in this study. To determine accidental findings and associated referral rates, researchers examined 1,260 CBCT interpretative reports from the Advanced Imaging Centre at the University of Alberta. Authors proposed a hierarchical transformer based model for report generation. Authors used bottom up attention mechanisms for medical report generation. The first part of their work is involved with the identification of visual features from CBCT scans with the help of bottom up attention implementation. The second part of the work deals with top down attention mechanism with the use of non recurrent captioning decoder. They claimed 50% better performance for BLEU-1 as compared to other systems under consideration. [4].

Keren Shemtov-Yona (2021),In this review discussed that the bone density, a property that can be challenging to assess, is used to qualitatively classify bone quality. Modern implantology, which depends on cutting-edge testing methods, requires a more precise assessment of bone quality. The databases that were used are PubMed and EMBASE. Clinical and radiological investigations detailing the proportions and characteristics of the alveolar ridge in various dentate and edentulous jaw regions were considered. A meta-analysis was conducted with the use of Alveolar ridge and its parameters and were reported as a composite mean using random effect models. To find variations in those outcome characteristics, meta regression statistical tests were carried out. Alveolar ridge dimensions and characteristics were analysed using CBCT and live human participants in the bulk of the chosen research (a total of 27). A typical alveolar ridge portrait was created using the meta-analysis's pooled mean. Quantitative bone classification based on geometry was suggested. It was discovered that the quantitative categorisation matched the current qualitative classification[5].

Hamdan S Alghamdi(2018) in this research delineated the techniques used to improve the osseointegration of dental implants in low-quality (type-IV) bone.These days, dental implants are a more popular method of replacing lost teeth with the goals of enhancing physical health, aesthetics, and chewing efficiency. Introduced by Brånemark in 1965, dental implants' robust osseointegration has been credited with their favourable clinical performance. The clinical result of implant treatment is complicated in impaired (bone) situations, which are commonly found in elderly people, even if the survival rate of dental implants during a 10-year observation has been reported to be higher than 90% in completely edentulous jaws. The biomechanical properties of bone in elderly persons do not provide adequate stability for implants; they are comparable to type-IV bone (classified by Lekholm& Zarb), where a reduced clinical fixation of implants has been amply proven. However, in order to create the latest advancements in contemporary dental implants, the quest for better osseointegration has persisted. This is a progression of findings from over two decades of study on implant-related aspects, such as surface characteristics, implant design, and surgical methods[6].

Zorrillaet.al(2024) in this study discussed the classification of bone tissue properties and how to evaluate them in dental implant design and placement investigations. Cone beam computed tomography has emerged as a popular implant planning technique. But in the majority of situations, not all the information is utilised, and the radiographic assessment of bone quality frequently relies on the clinician's subjective judgement. Using the "Bone quality, bone quantity, bone density, cone-beam CT, and cone-beam computed tomography." index phrases, three databases (Pubmed, Scopus, and Web of Science) were searched: Titles and abstracts were chosen by three reviewers based on inclusion and exclusion criteria. A few descriptions of the quality, density, and quantity of bone tissue that were used prior to or during the implantation of dental implants were chosen and grouped.Different classification schemes are currently used for dental implant planning and placement, especially when it comes to whether the quantity or quality of bone influences treatment results. The majority of authors, however, have not confirmed the classification's repeatability and diagnostic accuracy. In order to properly describe bone tissue assessment techniques, their diagnostic accuracy, and observer performance, a classification system that is in line with bone tissue properties must be developed[7].

Wang et.al (2023) in this research classified cortical and cancellous bones independently and used cone-beam computed tomography (CBCT) image processing for quantitative analysis, proposing a new way of identifying bone amount and quality at the dental implant site.CBCT preoperative images were acquired from 315 locations and 128 implant patients. Measure the cancellous bone density [in greyscale values (GV) and bone mineral density (g/cm3)] at the implant sites first, followed by the crestal cortical bone thickness (in mm). This study proposes a new "nine-square division" bone classification system for bone quality at the implant site. The cortical bone thickness is divided into three categories: A: > 1.1 mm, B: 0.7–1.1 mm, and C: < 0.7 mm. The cancellous bone density is divided into three categories: 1: > 600 GV (= 420 g/cm3), 2: 300–600 GV (= 160 g/cm3–420 g/cm3), and 3: < 300 GV (= 160 g/cm3). The components that were missed by earlier bone categorisation techniques (bone types A3 and C1) can be supplemented by the suggested classification[8].

Agrawal et.al(2021) in this study delineated the association and computations of bone mineral density (BMD), or the marginal bone loss from grey value, with evaluations of implant stability and bone quality. Implant dentistry has emerged as a viable option for replacing lost teeth in the modern period. A number of patient-related and procedure-dependent characteristics are necessary for the therapy process to be successful. Planning the surgical operation, choosing the right implant type, and ensuring the implant's effectiveness all depend heavily on the amount and quality of accessible bone. Implant stability, which has two levels—primary and secondary stability—predicts osseointegration. Primary stability is linked to Primary stability is linked to an implant's mechanical interlocking with the surrounding bone, which is influenced by the kind of implant (length, width, and surface texture), surgical technique, and bone quantity and quality. Biological stability through bone remodelling and regeneration is known as secondary stability, and it is influenced by primary stability. The following categories apply to factors influencing implants' primary stability: variables that are related to the patient (bone quantity and quality) and those that are not (implant design and surgical procedure).It has been discovered that the early bone loss in effectively osseointegrated implants typically begins at the crestal level, regardless of the surgical techniques (submerged or nonsubmerged). Bone mineral density (BMD) correlation and calculations are presented in this review, or the marginal bone loss from grey value using evaluations of implant stability and bone quality[9]

Troiano et.al (2024) in this research determined whether the Lekholm and Zarb classification system (L&Z) is reproducible for assessing the bone quality of edentulous alveolar ridges and to explore the possibilities of a data-driven approach to bone quality classification.Twenty-six skilled medical professionals were requested to use the L&Z categorisation (T0) to categorise 110 CBCT cross-sections. After a month, the same assessment was conducted again, however this time the images were arranged differently (T1). Cohen's kappa coefficient (CK) and Fleiss' kappa coefficient (FK) were used for intra- and inter-examiner agreement analyses, respectively. Furthermore, 3D edentulous ridge blocks produced from the same 110 CBCTs were subjected to radiomic feature extraction, and patterns in the collected data were found by unsupervised clustering utilising three distinct clustering techniques.T0 and T1 had a poor intra-examiner agreement (CK 0.515). There was very little inter-examiner agreement at any time (FK at T0: 0.273; FK at T1: 0.243). The three sets of 110 CBCTs were aggregated in the same manner by the three distinct unsupervised clustering techniques based on radiomic characteristics.Low clinician agreement with L&Z classification was found in the data, suggesting that the system may not be as trustworthy as previously believed. The current work raises the possibility of using a repeatable, data-driven method based on radiomics to classify edentulous alveolar ridges, which could have an impact on bettering clinical results. More investigation is required to ascertain the therapeutic relevance of these results and to create more precise and standardised techniques for evaluating the bone integrity of edentulous alveolar ridges[10].

Xie et.al(2024)in this paper suggested a completely automated tissue segmentation method for dental implant surgery. In particular,a data distribution histogram-based image preprocessing technique was presented that can adaptively handle CBCT images with various parameters. In light of this,the bone segmentation network was utilized to acquire the segmentation outcomes of the maxillary sinus, teeth, and alveolar bone. The relevant tasks were accomplished by using the tooth and mandibular regions as the ROI regions of tooth segmentation and mandibular nerve tube segmentation, respectively. The order information of the dentition can be obtained from the findings of tooth segmentation. Comparing proposed method to other approaches, the related experimental findings demonstrate that it can achieve superior segmentation efficiency and accuracy. 96.5%, 95.4%, 93.6%, and 94.8% were its average Dice scores for the tooth, alveolar bone, maxillary sinus, and mandibular canal segmentation tasks, respectively[11].

## 2.2	Importance of the proposed project in terms of impact and innovation

The major highlight in this proposed project is the generation of automated medical reports using Artificial Intelligence (AI) for CBCT scans meant for dental images. These images are used for the planning of tooth implantation process. Previously, this process required manual interference by the clinicians or experts. That manual process was time consuming and majorly depends upon the availability of the expert. The main role of clinicians or experts in this process is to identify manually the presence of some specific points on these CBCT images such as the crest, sinus floor, nasal floor, and inferior alveolar nerve (IAN) canal etc. This unavailability of the service not only affect the planning process but also faces the difficulties in the areas where there is no availability of the experts.

For generating automated medical reports for tooth implantation planning, AI based algorithms and model will be used for the purpose of pre-processing of input images, segmentation at different levels start from the Jaw segmentation till the tooth and bone segmentation. The proposed system will automate the identification of critical regions of interest within CBCT scans. The aim is to measure the quantitative distances form regions like crest to sinus floor and crest to IAN canal, which are crucial for implant placement prediction. These quantitative values further will be used for standard clinical classifications like Lekholm & Zarb and Misch, allowing for a qualitative analysis of bone density and structure. The final output will be a structured, user-friendly report that can be easily interpreted by both clinicians and patients.

The innovative aspect of this project lies in its ability to generate comprehensive medical reports without human intervention in context of dental CBCT scans. The major aspect related to this project is the savage of time during the report generation process, ability to generate reports without human intervention, not depending over the availability of clinicians or experts and also able to reduce the workload over the medical team. This overall process enable the experts to go for quick decision making.

In terms of societal impact, the system has the potential to democratize access to high-quality dental care by enabling general practitioners or smaller clinics without specialized radiologists to perform accurate implant planning. Additionally, the solution paves the way for real-time second opinions and remote diagnosis, which are especially beneficial in underserved areas.

In summary, the proposed project brings together cutting-edge AI techniques, domain-specific medical knowledge, and patient-centric design to address critical gaps in the current dental implant planning workflow. Its impact is far-reaching, promising innovation in both clinical practice and patient care in dental radiology and oral implant.

## 2.3       Proof of concept can be shared with appropriate preliminary data: NA

## 2.4	If the project is location specific, basis for selection of location be highlighted  : NA

# WorkPlan:

The following interrelated phases will be used to carry out the project:

Phase-1: Data Acquisition: The collection of CBCT scans and reports is required to be done in this phase. This is the most crucial phase of the proposed project. The first milestone in this phase is make an agreement with the medical clinician or the data center from which the CBCT scans needs to be collected. Milestone-II of this phase is to collect 1200 CBCT scans of different patient which fall under the different levels of lekholm and zarb or Mich classifications. This phase also deals with the pre-processing task on these collected scan to handle the issue of poisonous noise, Thermal noise and the Ring Artifacts produced during the scanning process under CBCT scans.

Phase-2: Model Development and Training: During this phase we will develop a model to deal with the process of segmentation and identification of the different regions of interests. The segmentation needs to be done to locate the regions of lower jaw and upper jaw means Mandible and Maxilla. After that the segmentation needs to be carried out at the level of tooth identification and bone density identification. The Milestone-1 in this phase deals with the implementation of self training algorithm to extract the regions of Mandible and Maxilla. After this the identification of tooth under consideration along with the information about its particular section in CBCT scan is performed. Milestone-2 under this phase is to train the system over CBCT reports considered for the training phase. Milestone-3 under phase-2 is to evaluate the model as per different validation sets.

Phase-3: User study and Evaluation: This phase is involved with the evaluation part of identified points in tooth and bone regions such as Nasal Floor, IAN Cannel, Crest regions. This first milestone is to find out the quantitative measures as the heights form crest to nasal floor, crest to IAN floor etc. These heights and the desired sections of the tooth under consideration will be identified. The milestone-2 in this phase is to extract the information about the qualitative measures like Lekholm and Zarb or Mich classifications. This leads to the idea about the qualitative measure of the bone quality to find the status of the implantation process or the possibility of the implantation.

Phase-4: Dissemination and Reporting: This phase is involved with the writing of the final report after the compilation of all the results and findings. This process involved with the publication of articles related to the findings of different phases for the sake of approval from the publishing agencies and exposure of the findings to the outside word.

Figure-1: Work-Flow of proposed project

Figure-1 represents the technical work flow of the proposed project. During phase-1 the project will start from the collaboration with clinical institute from which the data regarding the CBCT scans is required. This required data include both the soft copies of CBCT scans as well as the PDF files of the medical reports containing the quantitative parameters for these dental scans. During the collection process the pre-processing step will be performed while storing the images in the repository. This will enhance the overall quality of the stored images before going for further operations like segmentation and classification. Three common type of noise poisonous, thermal and ring-artifacts which may be there in a CBCT scan due the quality of the acquisition process will be handled during the pre-processing phase. The next phase will be involved with the development of self training algorithm. This algorithm will be used to segment various parts from the CBCT images. It will mainly involve with the extraction of the idea about the mandible and maxilla regions. Then the segmentation of designation tooth will be performed alone with the extraction of part representing bone area for that tooth under consideration. The CBCT reports containing the quantitative information in the form of PDF files will also be processed during this phase for the purpose of generation of training and testing sets data.  Phase-3 of the project will be involved with the identification of certain points of interests such as nasal floor, sinus floor, crest, IAN canal etc. These points will further help in the calculation of different height factors present in the CBCT images to analyze the quality of tooth and bone considered for the purpose of implantation. These heights will further help in assessing the overall quality of as per Lekholm and Zarb or Mich classifications. Finally the proposal reflects the requirement of the phase to complete the proper documentation required for the project along with writing the quality manuscripts to ensure the validity of the completed work.

## Suggested Plan of action for utilization of research outcome for social economic growth benefits expected from the project.

## As a research outcome of this project for social economic growth, it results in clinical integration and automation in dental practices by providing a novel solution to generate automatic medical reports by providing the CBCT image input. This will further help in reducing the cost of the overall process by removing the human intervention based over the expert in this field and demanding the high consultancy fees. The other impact will the huge reduce in waiting time for the generation of reports and the dependency over the availability of the expert to generate these reports by using the manual process. The other major impact of this project will be the increase in the accuracy of the system and the availability of a decision support system. This will lead to less failure rates during the treatment and will increase the patient's trust over the treatment process. It also leads to the skill improvement by using AI tools. This further help in the generation of new job opportunities in health care or dental treatment sector. One of the outcome of this project will be the creation of data repository for dental treatment. This further will help the new researchers to utilize this repository for their future research and guidance.

Environmental impact assessment and risk analysis.

Environmental Impact Assessment:

- During data acquisition phase, the usage of only the digital records like CBCT Scans and PDF reports

- No involvement of any physical waste generation

- Removal of noise at the time of pre-processing

- During model development and training the optimized use of resources.

- Secure data sharing

- Digital publications and open availability of the same for other researchers

  - Risk Analysis:

  - Data privacy and Compliance Risk (Mitigation: The implementation of secure storage protocols)

  - The delays in clinical partnerships (Mitigation: Early-stage MoUs and institutional ethics board clearance)

  - The reliability issues of the model(Mitigation: Inclusion of diverse data across patient types, and rigorous model validation strategies including cross-validation and expert feedback loops.)

  - Dissemination plan:

  - Publication of the work in professional conferences and Journals.

  - Patent Filing for the novel algorithms and the resultant software tool for the generation of automatic CBCT medical reports

  - Targeting the conduct of workshops in the host institute to spread the knowledge to the intended users

  - Book chapters contributions

## Time Schedule of activities giving milestones through BAR diagram.

| Phases | Year-1 | Year-2 | Year-3 |
| --- | --- | --- | --- |
| Phase-1: Data Acquisition | Q1-Q4 | Q1-Q2 | Q1-Q2 |
| Phase-2: Model Development and Training | Q3-Q4 | Q1-Q3 |  |
| Phase-3: User study and Evaluation |  | Q1-Q4 | Q1-Q4 |
| Phase-4: Dissemination and Reporting |  | Q4 | Q1-Q4 |

Bibliography

  - Kaaber L,  Matzen L.H, Schropp L, Spin-Neto R,"Low-dose CBCT protocols in implant dentistry: a systematic review",Oral Surgery, Oral Medicine, Oral Pathology and Oral Radiology, Volume 138, Issue 3,  2024, Pages 427-439, ISSN 2212-4403, https://doi.org/10.1016/j.oooo.2024.03.013.

  - Saha N,  Nair V., Role of CBCT in Dental Implant Treatment Plan: A Review, Journal of Medicine and Health Research, Volume 8, Issue 2, Page 1-5, 2023; Article no.JOMAHR.11456,  ISSN: 2456-9178.

  - Mikic, M.; Vlahovic, Z.; Stevanović, M.; Arsic, Z.; Mladenovic, R. The Importance of Correlation between CBCT Analysis of Bone Density and Primary Stability When Choosing the Design of Dental Implants—Ex Vivo Study. Tomography 2022, 8, 1293-1306. https://doi.org/10.3390/tomography8030107

  - Xiong, Y., Du, B., Yan, P. (2019). Reinforced Transformer for Medical Image Captioning. In: Suk, HI., Liu, M., Yan, P., Lian, C. (eds) Machine Learning in Medical Imaging. MLMI 2019. Lecture Notes in Computer Science(), vol 11861. Springer, Cham. https://doi.org/10.1007/978-3-030-32692-0_77

  - K. S. Id, “Quantitative assessment of the jawbone quality classification : A meta-analysis study,” 2021.

  - H. S. Alghamdi, “Methods to improve osseointegration of dental implants in low quality (type-IV) bone: An overview,” Journal of Functional Biomaterials, vol. 9, no. 1. MDPI AG, Jan. 13, 2018. doi: 10.3390/jfb9010007.

  - J. J. Palomino-Zorrilla, N. E. Córdova-Limaylla, J. C. Rosas-Díaz, C. F. Cayo-Rojas, L. A. Cervantes-Ganoza, and M. E. Guerrero, “Jawbone quality classification in dental implant planning and placement studies. A scoping review,” J. Int. Soc. Prev. Community Dent., vol. 14, no. 2, pp. 89–97, 2024, doi: 10.4103/jispcd.jispcd_4_22.

  - S.-H. Wang, J.-T. Hsu, L.-J. Fuh, S.-L. Peng, H.-L. Huang, and M.-T. Tsai, “New classification for bone type at dental implant sites: a dental computed tomography study,” BMC Oral Health, vol. 23, no. 1, 2023, doi: 10.1186/s12903-023-03039-2.

  - U. Agrawal, M. Jalaluddin, D. Roy, and R. Awasthi, “Is Bone Density a Success Criteria in Dental Implants,” Acta Sci. Dent. Scienecs, vol. 5, no. 6, pp. 146–150, May 2021, doi: 10.31080/asds.2021.05.1134.

  - G. Troiano et al., “Inter and intra-operator reliability of Lekholm and Zarb classification and proposal of a novel radiomic data-driven clustering for qualitative assessment of edentulous alveolar ridges,” Clin. Oral Implants Res., 2024, doi: 10.1111/clr.14271.

  - Y. Liu, “Fully automatic AI segmentation of oral surgery-related tissues based on cone beam computed tomography images,” no. March, 2024, doi: 10.1038/s41368-024-00294-z.

# Expertise:

## Expertise available with the investigators in executing the project:

Prof. (Dr.) Gurpreet Singh, the Principal Investigator (PI) of the proposed project, brings a wealth of experience and expertise highly relevant to the successful execution of this research. He possesses over 18 years of teaching and research experience in the field of Artificial Intelligence (AI) and Machine Learning (ML), with a strong interdisciplinary focus on medical data analysis and intelligent systems. Dr. Singh has an impressive research portfolio, including more than 40 publications in Scopus-indexed journals and conferences, reflecting his deep involvement in cutting-edge AI/ML applications and research dissemination. He has also been granted two patents, demonstrating his ability to translate theoretical innovations into practical, impactful solutions.

His expertise specifically aligns with the objectives of this project, such as:

- Image processing and segmentation techniques applied to CBCT scans,

- Development of AI/ML-based models for region identification and classification tasks,

- Implementation of self-training and supervised learning algorithms for automated decision systems,

- And a thorough understanding of data handling, annotation, validation, and model evaluation.

## Summary of roles/responsibilities for all Investigators:

| S. No. | Name of the Investigators | Roles/Responsibilities | Specific part of proposals where PI/Co-PI Contribution |
| --- | --- | --- | --- |
| 1. | Prof. (Dr.) Gurpreet Singh | PI | Lead authority and overall supervision |
| 2. | __ | JRF | Carrying day to day research activities |

## Key publications published (with impact factor) by the Investigator spertaining to the theme of the proposal during the last 5 years

Papers:

- Singh, G., Sachan, M.K. An Unconstrained and Effective Approach of Script Identification for Online Bilingual Handwritten Text. Natl. Acad. Sci. Lett. 43, 453–456 (2020). https://doi.org/10.1007/s40009-020-00889-0

- R. Kaur and G. Singh, "Performance Comparison of AI Models for Digital Image captioning," 2022 International Conference on Computational Modelling, Simulation and Optimization (ICCMSO), Pathum Thani, Thailand, 2022, pp. 257-261, doi: 10.1109/ICCMSO58359.2022.00058.

- Singh, G., & Singh, B. (2011). Feature based method for human facial emotion detection using optical flow based analysis. An International Journal of Engineering Sciences, 4(1), 363-372.

- Sekhon, N.K., Singh, G. (2023). Hybrid Technique for Human Activities and Actions Recognition Using PCA, Voting, and K-means. In: Gupta, D., Khanna, A., Hassanien, A.E., Anand, S., Jaiswal, A. (eds) International Conference on Innovative Computing and Communications. Lecture Notes in Networks and Systems, vol 492. Springer, Singapore. https://doi.org/10.1007/978-981-19-3679-1_27

- Verma, Divya, Gurpreet Singh and Hatesh Shyan. "Multilayer Convolutional Neural Network for Plant Diseases Detection." International Journal of Engineering and Advanced Technology (IJEAT) 9.5 (2020).

- R. Kaur and G. Singh, "Automatic Image Captioning for Medical Diagnosis Report Generation," 2023 11th International Conference on Intelligent Systems and Embedded Design (ISED), Dehradun, India, 2023, pp. 1-6, doi: 10.1109/ISED59382.2023.10444545.

Patent:

- Singh, J. P., Singh, S., Kusla, V., Singh, G., Mittal, S., Kaur, R., & Mamta. (2023). Development of wireless resource and computation offloading for enhanced energy efficiency in the Internet of Vehicles (IoV). Indian Patent Application No. 202311061436. Filed 12 September 2023. Published 13 October 2023.

- Gaur, P. K., Kaundal, M., Sharma, A., Singh, G., Bhatia, V., & Chauhan, R. S. (2021). Air bag for train [Design registration]. Design No. 354845-001. Registered 12 December 2021. India.

List of funded projects implemented/being implemented: NA

| S.No | Project Title | Funding Agency | Budget | Duration | Start date & EndDate (MM/YYYY) | Status (in Chronolog ical order) (Ongoing/ completed/ Submitted) | Role as PI/ Co- PI | PCR evaluation grade it funded by (ANRF/ SERB) | Summa ry of Result & Publica tion/ Patent |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# List of facilities available at host institute

## Infrastructural Facilities

| Sr. No. | Infrastructural Facility | Yes/No/Not required Full or sharing basis |
| --- | --- | --- |
| 1. | Workshop Facility | Yes |
| 2. | Water & Electricity | Yes |
| 3. | Laboratory Space/Furniture | Yes |
| 4. | Power Generator | Yes |
| 5. | AC Room or AC | Yes |
| 6. | Telecommunication including e-mail & fax | Yes |
| 7. | Transportation | Yes |
| 8. | Administrative/Secretarial support | Yes |
| 9. | Information facilities like Internet/Library | Yes |
| 10. | Computational facilities | Yes |
| 11. | Animal/Glass House | Not required |
| 12. | Any other special facility being provided | Not required |

  - Equipment available with the Institute/Group/Department/Other Institutes for the project:

| Equipment available with | Specific Name of Equipment & detailed description | Model, Make & year of purchase | Remarks including accessories available and current usage of equipment |
| --- | --- | --- | --- |
| PI&hisgroup |  |  |  |
| PI'sDepartment | __ | __ | __ |
| OtherInstitute(s) in the region |  |  |  |

(List of equipment's available at the investigator's and co-investigator's institute and at the department. Also list of infrastructure details provided by the host institute and at the co- investigator's institute. (It is mandatory -Please use separate sheet)

| Equipmentavailable with | Specific Name of Equipment & detaileddescription | Model, Make & yearofpurchase | Remarksincludingaccessories available and current usage of equipment |
| --- | --- | --- | --- |
| Co-PI&hisgroup |  |  |  |
| Co-PI'sDepartment |  |  |  |
| OtherInstitute(s) inthe region |  |  |  |

- Name and address of experts/institution interested in the subject/outcome of the project. Not Yet Approached

| S.No. | Name of Expert Affiliation | Email | Area of Expertise |
| --- | --- | --- | --- |

- Current financial support, other than those mentioned in section 4.4. Nothing yet

- Any other relevant information & supplementary documentation. Not required